Supplementary Material S1

Representative MARG Benchmark Queries for SHM Lifecycle Memory

Signal-Driven Lifecycle Memory for SHM Digital Twins: A MARG Framework with Physics-Informed Prediction and Multi-Infrastructure Validation

Dong-Wook Kim  |  DL E&C Co., Ltd., Seoul  |  clearup7@nate.com  |  ORCID: 0000-0002-0721-5114

Purpose. This Supplementary Material provides 40 representative benchmark queries drawn from the held-out test set (320 queries, 80 per memory type) used to evaluate MARG retrieval performance in the main manuscript (Section 5.5, Table 8). These 40 queries constitute a representative reproducibility subset supporting inspection and partial verification of the retrieval evaluation protocol reported in the main manuscript. The full test set (κ = 0.83 inter-rater agreement) is available from the corresponding author upon reasonable request.

Query Structure. Each query record includes: (1) Query ID (format Q-[MemType]-[nn]), (2) case study identifier (A = Railway viaduct, B = Expressway, C = PSC bridge), (3) natural-language maintenance query, (4) memory type required, (5) required evidence types, (6) relevant record IDs from the LTCM Knowledge Graph (anonymised), and (7) ground-truth answer established by three senior maintenance engineers (κ = 0.83 inter-rater agreement).

Query Distribution. 10 Episodic (E), 10 Semantic (S), 10 Procedural (PR), 10 Predictive (PD). Case A: 14 queries; Case B: 12 queries; Case C: 14 queries. Multi-hop queries requiring joint retrieval from ≥2 memory types are marked in the Query column with an asterisk (*) in the full test set; this subset includes representative examples from each type.

Note on Record IDs. Record IDs follow the LTCM naming convention [Case]-[Type]-[Year]-[Location]. Full Knowledge Graph (321,700 RDF triples) and IFC 4.3-to-RDF mapping are documented in Appendix A of the main manuscript. Where necessary for anonymisation, case-specific operational identifiers have been normalised while preserving the decision logic used in the evaluation. Design code thresholds (KR C-14020, KDS 24 10 20, MOLIT specifications) are cited from the referenced Korean standards.

Table S1.  Representative MARG Benchmark Queries — 40 Queries across 4 Memory Types and 3 Infrastructure Case Studies.

| Query ID | Case | Query | Memory Type | Required Evidence | Relevant Record IDs | Ground Truth |
| --- | --- | --- | --- | --- | --- | --- |
| Q-E-01 | A | What were the peak track settlement measurements on Span 7 between 2018 and 2021? | Episodic | Bi-annual geometric survey records; settlement sensor logs | A-GEO-2018-S7, A-GEO-2019-S7, A-GEO-2020-S7, A-GEO-2021-S7 | Settlement increased from 6.2 mm (2018) to 11.4 mm (2021); rate 1.73 mm/yr |
| Q-E-02 | A | List all emergency maintenance events on the railway viaduct from 2012 to 2024. | Episodic | Work order database; maintenance log | A-WO-2014-EMG, A-WO-2016-EMG, A-WO-2019-EMG×3, A-WO-2021-EMG×2 (6 records) | 6 emergency closures; 2019 cluster (3 events) triggered by differential settlement >3 mm |
| Q-E-03 | A | Which IoT sensors on Span 9 recorded anomalies exceeding 2σ threshold in 2022? | Episodic | IoT anomaly records; sensor metadata | A-IOT-2022-S9-ACC03, A-IOT-2022-S9-STR07, A-IOT-2022-S9-SET02 | 3 sensors; accelerometer ACC03 (vertical vibration), strain gauge STR07, settlement sensor SET02 |
| Q-E-04 | A | What repair interventions were performed on Track Sections 3–6 after the 2021 inspection? | Episodic | Work order records; inspection reports post-2021 | A-WO-2021-SEC3, A-WO-2022-SEC4, A-WO-2022-SEC5, A-WO-2022-SEC6 | Tamping (Sec 3, Nov 2021), rail grinding (Sec 4–5, Mar 2022), ballast renewal (Sec 6, Jul 2022) |
| Q-E-05 | B | Retrieve all pothole repair records on Section 4 expressway from 2020 to 2023. | Episodic | Road maintenance database; GPS-tagged repair records | B-REP-2020-S4×14, B-REP-2021-S4×28, B-REP-2022-S4×19, B-REP-2023-S4×11 (72 records) | 72 events; highest density in 2021 (28 events); km 14.3–16.7 hotspot cluster |
| Q-E-06 | B | What was the PCI score trend for km 10–15 on the expressway from 2016 to 2022? | Episodic | Profilometer survey records; FWD data | B-PCI-2016-KM10-15, B-PCI-2018-KM10-15, B-PCI-2020-KM10-15, B-PCI-2022-KM10-15 | PCI: 84 (2016) → 78 (2018) → 69 (2020) → 58 (2022); avg degradation 4.3 PCI/yr |
| Q-E-07 | C | Show the complete bearing replacement history for Span 2 of the PSC bridge. | Episodic | Bearing replacement log; cost database | C-BR-1998-SP2A, C-BR-2003-SP2B, C-BR-2009-SP2A, C-BR-2017-SP2B, C-BR-2022-SP2A | 5 replacements (1998–2022); avg service life 12.4 yr; cost trend: 7.2M → 9.8M → 12.1M → 14.3M → 16.7M KRW |
| Q-E-08 | C | Find all Grade-D corrosion inspection findings on the PSC bridge from 2010 to 2020. | Episodic | Visual inspection reports; corrosion grade records | C-INS-2012-GD, C-INS-2015-GD×2, C-INS-2018-GD×3, C-INS-2020-GD×2 | 8 Grade-D findings across Spans 1–4; cluster in 2018 (3 findings on Spans 2–4) |
| Q-E-09 | C | What were the load cell readings for Span 4 bearing 4B during the 2019 inspection? | Episodic | Load cell measurement records; inspection metadata | C-LC-2019-SP4B | Vertical load: 2,847 kN (92% of design capacity); horizontal eccentricity: +3.2 cm (exceeds ±2 cm limit) |
| Q-E-10 | A | Retrieve the three most recent anomaly events on Spans 7–12 sorted by severity. | Episodic | IoT anomaly records; severity scores | A-IOT-2023-S09-SET (sev=0.82), A-IOT-2023-S11-ACC (sev=0.74), A-IOT-2022-S07-STR (sev=0.68) | Top-3: Span 9 settlement (2023, sev=0.82), Span 11 acceleration (2023, sev=0.74), Span 7 strain (2022, sev=0.68) |
| Q-S-01 | A | What is the KR C-14020 threshold for absolute track settlement and differential settlement? | Semantic | Korean railway design code ontology node | S-KR-C14020-SETTLE | Absolute: 15 mm; Differential (adjacent spans): 3 mm |
| Q-S-02 | A | What are the material properties of PSC track slab concrete used in this viaduct? | Semantic | Material specification ontology; design specification records | S-MAT-PSCCONC-C60 | Concrete grade C60; f'c=60 MPa; Ec=38.2 GPa; carbonation rate k=0.31 mm/yr^0.5 (KR exposure class XC3) |
| Q-S-03 | B | What degradation model governs pavement PCI decay for flexible pavement under ESAL loading? | Semantic | Pavement degradation ontology; KPAVE model node | S-DEG-PCI-ESAL | ΔPCI = f(ESAL, layer stiffness moduli); empirical Pavement ME Design model; layer moduli from FWD backcalculation |
| Q-S-04 | B | What inspection frequency does MOLIT require for expressway pavement PCI below 60? | Semantic | MOLIT road management specification ontology | S-MOLIT-PCI-INSP | PCI 50–60: semi-annual profilometer survey + annual FWD; PCI <50: quarterly + immediate structural review |
| Q-S-05 | C | What is the KDS 24 10 20 specification for bridge bearing service life and replacement criteria? | Semantic | Korean bridge design standard ontology node | S-KDS-241020-BEAR | Design service life: 20 ± 3 yr; replacement criteria: corrosion Grade D + vertical deformation >10% of design height; coastal exposure: service life multiplied by 0.71 (factor 1/1.4) |
| Q-S-06 | C | What failure mode ontology class covers PTFE sliding layer wear in elastomeric bearings? | Semantic | LTCM Infrastructure O&M Ontology; failure mode class hierarchy | S-FM-BEAR-PTFE-WEAR | Class: ltcm:SlidingLayerWear (subclass of ltcm:BearingWear); Archard wear law governs; triggers: load ×velocity; detection: displacement sensor + visual inspection |
| Q-S-07 | A | What is the relationship between Sato–Odaka parameters α and β for ballasted track? | Semantic | Track geometry degradation ontology; Sato-Odaka parameter node | S-DEG-SATOODAKA-PARAM | Typical ranges: α ∈ [0.9, 1.5] mm/cycle^β; β ∈ [0.68, 0.75]; α correlates with axle load, β with ballast gradation. Case A fitted: α=1.21, β=0.72. |
| Q-S-08 | B | What are the sub-base failure mode categories in the LTCM road infrastructure ontology? | Semantic | Road infrastructure ontology; failure mode hierarchy | S-FM-ROAD-SUBBASE | 3 categories: (1) moisture ingress (softening), (2) differential frost heave, (3) fatigue cracking from ESAL. Section 4 primary mode: moisture ingress (GPR-confirmed 2020). |
| Q-S-09 | C | What corrosion mechanism governs steel anchor bolt failure in bridge bearings? | Semantic | Steel corrosion ontology; anchor bolt failure mode | S-FM-BEAR-ANCHORSTEEL | Electrochemical corrosion under marine exposure; chloride penetration depth governs; depassivation threshold: [Cl⁻] > 0.4% by cement weight. Coastal factor ×1.4 per KDS 14 31 20. |
| Q-S-10 | A | What sensor types are classified under structural deformation monitoring in the LTCM ontology? | Semantic | Sensor classification ontology; structural monitoring class | S-SENSOR-DEFORM | 4 types: (1) LVDT displacement, (2) inclinometer, (3) GPS settlement, (4) fibre-optic strain. Case A uses types 2, 3, 4 (312 sensor nodes). |
| Q-PR-01 | A | What is the standard inspection procedure for track settlement exceeding 12 mm on a PSC viaduct? | Procedural | KR C-14020 inspection procedure graph | PR-KR-SETTLE-L1 | Step 1: Bi-axial levelling survey within 7 days. Step 2: FEM update with measured settlements. Step 3: Traffic restriction if differential >2 mm. Step 4: Tamping works within 30 days. Step 5: Re-survey within 14 days post-intervention. |
| Q-PR-02 | A | What emergency protocol applies when IoT sensors detect differential settlement exceeding 3 mm in real time? | Procedural | Emergency response protocol graph; track safety SOP | PR-KR-EMGSETTLE | Trigger: Δsettlement > 3 mm between adjacent spans. Action: (1) Immediate speed restriction 25 km/h, (2) Operations centre alert within 15 min, (3) On-site inspection within 2 hr, (4) Closure decision within 4 hr. |
| Q-PR-03 | B | What is the standard pavement overlay selection procedure when PCI falls below 55? | Procedural | MOLIT pavement management procedure graph | PR-MOLIT-OVERLAY | Step 1: Core sampling (3 per 500 m). Step 2: FWD structural assessment. Step 3: If structural number OK → microsurfacing; if SN deficient → mill-and-fill; if sub-base failure → full-depth reclamation. Step 4: Traffic control plan. Step 5: Post-overlay PCI survey within 30 days. |
| Q-PR-04 | B | What documentation is required before initiating a full-depth pavement reclamation on an expressway? | Procedural | Expressway construction procedure; regulatory submission checklist | PR-EXPY-FDR-DOC | Required: (1) Traffic management plan approved by MOLIT, (2) Environmental impact notice (>5,000 m²), (3) Core sampling report, (4) Mix design specification, (5) Lane closure schedule with 14-day public notice. |
| Q-PR-05 | C | What is the planned bearing replacement procedure for Grade-D corrosion on an in-service bridge? | Procedural | KDS 24 10 20 bearing replacement procedure; bridge closure protocol | PR-KDS-BEAR-REPLACE | Step 1: Temporary shoring design and approval. Step 2: Lane closure — alternating half-width. Step 3: Hydraulic jack installation. Step 4: Bearing removal, surface preparation, new bearing installation. Step 5: Load redistribution verification. Step 6: Progressive load testing before full reopening. |
| Q-PR-06 | C | What inspection protocol applies when a bridge bearing shows sudden load transfer without prior warning? | Procedural | Bridge emergency inspection SOP; structural safety protocol | PR-BRIDGE-EMGBEAR | Classify as Level-3 emergency. Step 1: Immediate bridge closure. Step 2: Structural engineer on site within 1 hr. Step 3: Temporary shoring within 4 hr. Step 4: Cause investigation and FEM analysis within 48 hr. Step 5: Repair or closure decision within 72 hr. |
| Q-PR-07 | A | What are the steps for encoding a new anomaly event into the LTCM Episodic Memory? | Procedural | LTCM memory generation SOP; CLS-LTCM consolidation algorithm procedure | PR-LTCM-ENCODE | Step 1: Classify event by type (anomaly/inspection/work order). Step 2: Assign IFC 4.3 element GUID. Step 3: Extract signal metadata (timestamp, sensor ID, value, severity score). Step 4: CLS-LTCM significance scoring S(m). Step 5: If S(m)≥τ → retain as Episodic; else → consolidate into Semantic. |
| Q-PR-08 | B | What quality control checks are required after GPR sub-base assessment on the expressway? | Procedural | GPR QC procedure; MOLIT road inspection SOP | PR-GPR-QC | Step 1: Verify antenna frequency calibration (1 GHz for sub-base, 2 GHz for surface). Step 2: Validate against minimum 3 core samples per 1 km. Step 3: Check layer depth accuracy (tolerance: ±5 mm). Step 4: Generate layer thickness map. Step 5: Flag anomalies where measured depth deviates >20% from design. |
| Q-PR-09 | C | What maintenance procedure applies to an elastomeric bearing with PTFE sliding layer wear rate exceeding design limits? | Procedural | PTFE bearing maintenance procedure; KDS 24 10 20 wear limit | PR-BEAR-PTFE-02 | Step 1: Quantify wear depth (profilometry or thickness gauge). Step 2: If wear >30% of design thickness → scheduled replacement within 12 months. Step 3: If wear >50% → emergency replacement within 3 months. Step 4: Log replacement in episodic memory with grade, load, and cost. |
| Q-PR-10 | A | What is the re-inspection protocol after tamping works on a settlement-affected viaduct span? | Procedural | Post-tamping inspection SOP; KR C-14020 | PR-KR-TAMP-REINSP | Re-inspection at 7, 30, and 90 days post-tamping. If settlement recurrence >5 mm within 90 days → full ballast renewal. Effectiveness threshold: settlement rate <0.5 mm/month within 90-day monitoring period. |
| Q-PD-01 | A | What is the PINN-predicted probability that Span 9 settlement will exceed 15 mm within 18 months? | Predictive | PINN Sato-Odaka forecast model; current settlement record | PD-A-S09-SETTLE-2024 | P(settlement ≥ 15 mm ¦ 18 months) = 0.81; 95% CI: [0.69, 0.91]; Sato-Odaka: α=1.38, β=0.71; projected exceedance: 2025-Q2 |
| Q-PD-02 | A | Which spans of the railway viaduct are predicted to require tamping within the next 24 months? | Predictive | Multi-span PINN settlement forecast; maintenance planning model | PD-A-MULTISPAN-2024 | Priority 1 (0–12 months): Spans 9, 11. Priority 2 (12–24 months): Spans 7, 12. Spans 1–6, 8, 10, 13–18: no action needed. |
| Q-PD-03 | A | How does the PINN-predicted settlement rate change if axle load is increased from 25 t to 30 t? | Predictive | Sato-Odaka sensitivity analysis model; load-parameter relationship | PD-A-LOAD-SENS | Settlement rate increases 23% (Sato-Odaka α scales as α ∝ P^0.65); Span 9 threshold exceedance advanced from 2025-Q2 to 2024-Q4 under 30t loading. |
| Q-PD-04 | B | What is the predicted PCI for km 10–15 expressway in 2026 and 2028 under current maintenance? | Predictive | PCI degradation PINN forecast; maintenance scenario model | PD-B-PCI-FC2026-2028 | Under current maintenance: 2026 PCI = 49 (critical); 2028 PCI = 38 (failure range). Under proactive overlay (2025): 2026 PCI = 72; 2028 PCI = 64. |
| Q-PD-05 | B | Forecast remaining service life of the expressway sub-base on Section 4 without intervention. | Predictive | Sub-base structural degradation model; FWD structural number trend | PD-B-SUBBASE-RSL | Remaining service life: 3.2 years (95% CI: 2.1–4.4 yr); structural number will fall below minimum (SN=3.5) by 2027-Q1 based on FWD trend (ΔSN = −0.31/yr). |
| Q-PD-06 | C | What is the predicted remaining service life of Span 1A bearing based on the power-law corrosion model? | Predictive | Power-law bearing degradation PINN; Span 1A load cell history | PD-C-SP1A-BEAR-RSL | Remaining service life: 4.8 yr (95% CI: 3.2–6.4 yr); mass loss rate k=0.041 kg/yr, n=1.18; KDS design life not yet exceeded but coastal factor shortens projection. |
| Q-PD-07 | C | Rank all 10 bridge bearings by predicted remaining service life and recommended replacement year. | Predictive | Multi-bearing PINN forecast; bearing inventory | PD-C-ALLBEAR-RANK | Rank 1: SP2A (RSL 2.1 yr → replace 2026). Rank 2: SP4B (RSL 2.8 yr → replace 2026). Rank 3: SP3B (RSL 3.5 yr → replace 2027). Remaining 7: RSL >4 yr. |
| Q-PD-08 | C | How does PINN prediction accuracy change as the bearing episodic corpus grows from 5 to 14 events? | Predictive | PINN corpus-size sensitivity; prediction RMSE trend | PD-C-CORPUS-SENS | RMSE trend: 5 events = 3.8 yr; 8 events = 2.4 yr; 11 events = 1.8 yr; 14 events = 1.4 yr. Diminishing returns after 10 events; 63.2% RMSE reduction from 5 to 14 events. |
| Q-PD-09 | A | Generate a 5-year predictive maintenance plan for the railway viaduct based on PINN settlement forecasts. | Predictive | Multi-span settlement PINN; maintenance cost model; KR C-14020 thresholds | PD-A-5YR-PLAN | Year 1: Spans 9, 11 tamping (KRW 34M). Year 2: Spans 7, 12 tamping (KRW 28M). Year 3: Span 9 second tamping (KRW 18M). Year 4: Spans 14, 15 monitoring only. Year 5: Full section assessment. Total 5-yr cost: KRW 80M (vs. reactive scenario KRW 124M). |
| Q-PD-10 | C | What confidence interval does the PINN model assign to bearing replacement prediction for Span 2A in 2026? | Predictive | PINN power-law model; Monte Carlo Dropout uncertainty quantification | PD-C-SP2A-2026-CI | P(replacement needed by 2026) = 0.87; 95% CI from MC Dropout (T=100): [0.79, 0.93]; model uncertainty σ=0.031; main uncertainty source: corrosion rate k estimation (CV=18%). |

Abbreviations: E = Episodic Memory; S = Semantic Memory; PR = Procedural Memory; PD = Predictive Memory. Case A = Railway viaduct (4.2 km, 18 PSC spans); Case B = Expressway Section 4 (23 km); Case C = PSC bridge (210 m, 32-year record). PINN = Physics-Informed Neural Network; KG = Knowledge Graph; KDS = Korean Design Standard; KR = Korean Railway Standard; MOLIT = Ministry of Land, Infrastructure and Transport; PCI = Pavement Condition Index; ESAL = Equivalent Standard Axle Load; FWD = Falling Weight Deflectometer; GPR = Ground Penetrating Radar; RMSE = Root Mean Square Error; CI = Confidence Interval; RSL = Remaining Service Life.